# Homework 1

Giuseppe Labate 2095665

## Analytical solution

The analytic solution for a planar robotic arm with 3 revolute joints (3R) can be determined because the three joint axes are consecutive and parallel to each other.

Since the links dimensions are a1= a2=a3=1 the robot is built like in Figure 1

Figure 1: Schematic of the robot

The workspaces are:

- Reachable workspace: WS1=p∈R2:p≤3 ⊂ R2

- Dexterous workspace WS2=p∈R2:p≤1 ⊂ R2

Now the desired point is xed=210  with px = 2 and py = 1.
This means that the norm is given by  p= px2+ py2= 5.
By looking at the WS1 and WS2, we can say that the robot is able to reach xed but not in a dexterous way since xed∈WS1and  xed∉WS2.

By looking at the k(q) and xed, we can derive this system of equations  c1+c12+c123=2s1+s12+s123=1θ1+θ2+θ3=0
Using θ1+θ2+θ3=0 we get this system of equations:
c1+c12+ c0=2s1+s12+s0=1 →c1+c12=1s1+s12=1

### Find  θ2

To find θ2 we can square the two equations and sum the results.
(s1+s12)2+(c1+c12)2=2
s12+s122+2s1s12+c12+c122+2c1c12=2

By using the trigonometry fundamental identity s12+c12=1
	 2s1s12+2c1c12=0 		 s1s12+c1c12=0
s1s1c2+c1s2+c1(c1c2-s1s2)		c2s12+c12=0

Again, using the trigonometry fundamental identity:

c2=0			s2=± 1-c22=±1		θ2=ATAN2{s2, c2}

So, we get two results for θ2, which makes sense since we can reach the desired point with two different configurations.

θ2=±π2

### Find  θ1

To find θ1, we can replace θ2 in c1+c12=1s1+s12=1

c1+c1c2-s1s2=1s1+s1c2+c1s2=1

- If θ2=+π2
c1-s1=1s1+c1=1			s1+c1+c1-s1=2		c1=1

Replace c1=1 in one of the two equations: 
1+s1=1		s1=0
θ1=ATAN2s1, c1=0

- If θ2=-π2
s1+c1=1s1-c1=1			s1+c1+s1-c1=2		s1=1

Replace s1=1 in one of the two equations: 
1+c1=1		c1=0
θ1=ATAN2s1, c1=+π2

### Find  θ3

Finally, use the θ1+θ2+θ3=0 to find θ3.

- If θ1=+π2 and θ2=-π2
θ3=0

- If θ1=0 and θ2=+π2
θ3=-π2

### Results

The solutions for the inverse kinematic problem are:

- θ1=0,θ2=+π2,θ3=-π2

Figure 2a: First configuration

- θ1=+π2,θ2=-π2,θ3=0

Figure 2b: Second configuration

## Gradient Method

To solve the inverse kinematic problem using the gradient method, I’ve implemented a MATLAB script that applies the following formula until the error drops below a threshold (in my case I’ve set it to 1e-6).

qk+1=qk+αJkT qk xed-kqk

## If the error fails to fall below the threshold within 1000 iterations, we consider it a divergence.

## 2.1) Results for α=12

### 2.1.1) q0=000

Here the algorithm can’t find a proper solution since the value of α is too big.
As evidence of this we can see the q-values (Figure 3a) and the error (Figure 3b) keep bouncing back and forth.

Figure 3a: the values of q oscillate continuously, indicating that the chosen alpha is too large and we keep missing the solution eternally

Figure 3b: the error oscillates continuously, indicating that the chosen alpha is too large and we keep missing the solution eternally

### 2.1.2) q0=π2π2π2

What said above is true also for this initialization.

Usually we can start from different initializations to reach all possible solutions, but in this case, even if we’re starting from a different point q (0), alpha is still too large to fall into the minimum error and find the actual solutions to the problem.
As you can see, the solution (Figure 4a) and the error (Figure 4b) keep changing as before.

Figure 4a: q-values

Figure 4b: Error

## 2.2) Results for α=110

Since we’ve now lowered the alpha value, we can converge to a solution by approaching it gradually, unlike before, when the alpha was too large and the gradient method couldn’t converge to the minimum because it kept bouncing around it.

Note that we sacrifice computation time to ensure that the machine actually converges to a solution.
With alpha = 1/2, convergence would be much faster, but we wouldn’t be sure of reaching a solution at all.

### 2.2.1) q0=000

The solution obtained is: q=0π2-π2, so the configuration assumed by the arm is the one that we can see in Figure 2a

Figure 5: q-values on the left and the error on the right

### 2.2.2) q0=π2π2π2

Because of the small alpha, even when starting from a different value of q, we’re still able to reach a solution.
Usually, multiple different initializations are used to find the various solutions to the problem.
In this case, however, we end up finding the same solution as before.

q=0π2-π2

Figure 6: q-values on the left and the error on the right

## Newton Method

The Newton Method has the advantage of converging much faster than the Gradient Method.
However, it has a major drawback: if the robot starts from or passes through a configuration with a singular point, the method fails and cannot find a solution.
This happens because the method requires inverting the Jacobian J to compute the new q, as one can see in the formula below.
qk+1=qk+Jk-1qkxed-kqk  
When the robot reaches a singular point, the Jacobian cannot be inverted since its determinant becomes zero.
In this case, det( J )=1sin(θ2)​, so when θ2= 0 or θ2= π, the Newton Method cannot be used. 
To compute the solutions using the Newton Method, I wrote a MATLAB script that applies the formula, stops if it reaches a singular point, and accepts a solution only if the error is below a threshold of 1e-6.

### 3.1) q0=000

In this case, since θ2= 0, the robot arm is at a singular point, and the Newton Method fails → no solution.

## 3.2) q0=π2π2π2

Here, the method successfully converges to this solution: 
q=0π2-π2
Note that it takes only 6 iterations to converge, compared to the 700/800 required by the Gradient Method.

Figure 7: q-values on the left and the error on the right